Contoso Electronics

Equipment Reference & Maintenance Guide

Automated Optical Inspection System

Contoso VisionCheck AOI-500

Asset Identification

| Asset ID | CE-AOI-2400 |
| --- | --- |
| Category | Quality Inspection |
| Manufacturer | Contoso Vision Systems |
| Location | SMT Line 1 - Position 6 |
| Installation Date | 2022-02-18 |
| Warranty Expiry | 2027-02-18 |
| Support Contact | quality.support@contoso.com ¦ ext. 4620 |

Overview

The Contoso VisionCheck AOI-500 is an automated optical inspection system that detects assembly defects such as missing components, misalignment, solder bridges, and polarity errors immediately after reflow. It uses multi-angle lighting and AI-assisted defect classification to maximize detection while minimizing false calls.

Technical Specifications

| Parameter | Value |
| --- | --- |
| Camera Resolution | 18 µm/pixel |
| Lighting | Multi-angle RGB + coaxial |
| Inspection Speed | Up to 55 cm²/s |
| Min Component | 01005 |
| PCB Size | 50 x 50 to 510 x 460 mm |
| Defect Library | AI-assisted, self-learning |
| Power | 230 VAC, 50/60 Hz, 1.2 kW |

Preventive Maintenance Schedule

| Interval | Task |
| --- | --- |
| Daily | Clean camera window; verify reference board passes. |
| Weekly | Clean conveyor and lighting diffusers. |
| Monthly | Recalibrate camera and lighting intensity. |
| Quarterly | Update defect library; verify measurement gauge R&R. |

Safety Guidelines

Do not stare into inspection LEDs at close range.

Follow ESD procedures when handling boards.

Keep conveyor area clear during operation.

Power down before cleaning camera optics.

Common Issues & Troubleshooting

| Symptom | Recommended Action |
| --- | --- |
| High false-call rate | Retune thresholds; clean optics; verify lighting calibration. |
| Missed defects | Update defect models; check camera focus and resolution. |
| Board not detected | Verify conveyor sensors and fiducial recognition. |
| Inconsistent measurements | Recalibrate; check for ambient light interference. |

Contoso Electronics internal maintenance documentation. For service requests, contact quality.support@contoso.com | ext. 4620 or submit a work order through the maintenance portal.