Contoso Electronics

Equipment Reference & Maintenance Guide

Conformal Coating Machine

Contoso CoatShield CC-400

Asset Identification

| Asset ID | CE-CCM-2800 |
| --- | --- |
| Category | Finishing Line |
| Manufacturer | Contoso Finishing Systems |
| Location | Finishing Line 3 - Position 1 |
| Installation Date | 2022-04-27 |
| Warranty Expiry | 2026-04-27 |
| Support Contact | finishing.support@contoso.com ¦ ext. 4660 |

Overview

The Contoso CoatShield CC-400 is a selective conformal coating system that applies protective coating to PCBs to guard against moisture, dust, and chemical contamination. It uses programmable spray valves and vision for precise, masked-free selective coating.

Technical Specifications

| Parameter | Value |
| --- | --- |
| Coating Type | Acrylic, silicone, urethane |
| Applicators | Film-coat + needle valve |
| Positioning Accuracy | ±0.1 mm |
| Max PCB Size | 450 x 450 mm |
| Coating Speed | Up to 400 mm/s |
| Curing | Integrated UV / thermal option |
| Power | 230 VAC, 50/60 Hz, 3 kW |

Preventive Maintenance Schedule

| Interval | Task |
| --- | --- |
| Each shift | Purge and clean spray valves; check material level. |
| Daily | Inspect nozzles for clogging; verify spray pattern. |
| Weekly | Clean coating booth and exhaust filters. |
| Monthly | Recalibrate valves and vision; check UV lamp output. |

Safety Guidelines

Coating solvents are flammable - no open flames; ensure extraction.

Wear gloves and respiratory protection as required by SDS.

UV curing lamps emit hazardous UV - never look at the lamp.

Store coating materials per SDS requirements.

Common Issues & Troubleshooting

| Symptom | Recommended Action |
| --- | --- |
| Uneven coating | Clean/replace nozzle; verify viscosity and spray pressure. |
| Nozzle clogging | Purge system; use recommended thinner; check material shelf life. |
| Coating in keep-out areas | Recalibrate vision alignment; verify program pattern. |
| Incomplete cure | Check UV lamp hours/output or oven temperature and time. |

Contoso Electronics internal maintenance documentation. For service requests, contact finishing.support@contoso.com | ext. 4660 or submit a work order through the maintenance portal.