Contoso Electronics

Equipment Reference & Maintenance Guide

Reflow Soldering Oven

Contoso ThermFlow RO-800

Asset Identification

| Asset ID | CE-RFO-2100 |
| --- | --- |
| Category | SMT Line |
| Manufacturer | Contoso Assembly Tools |
| Location | SMT Line 1 - Position 5 |
| Installation Date | 2021-11-30 |
| Warranty Expiry | 2026-11-30 |
| Support Contact | smt.line.support@contoso.com ¦ ext. 4501 |

Overview

The Contoso ThermFlow RO-800 is an 8-zone convection reflow oven used to solder surface-mount assemblies by precisely following a thermal profile. It provides uniform heating, nitrogen inerting, and closed-loop conveyor control for high-yield SMT production.

Technical Specifications

| Parameter | Value |
| --- | --- |
| Heating Zones | 8 top + 8 bottom |
| Max Temperature | 350 °C per zone |
| Conveyor Width | 50 - 400 mm adjustable |
| Conveyor Speed | 0.1 - 2.0 m/min |
| Atmosphere | Air or Nitrogen |
| Profile Storage | 200 recipes |
| Power | 3-phase 400 VAC, 40 kW |

Preventive Maintenance Schedule

| Interval | Task |
| --- | --- |
| Daily | Check conveyor tracking and flux collection trays. |
| Weekly | Clean flux from heater zones and exhaust; inspect chains. |
| Monthly | Run profile verification with thermocouple board. |
| Quarterly | Replace flux filters; inspect blower motors. |

Safety Guidelines

Surfaces and boards exit the oven extremely hot - use heat-resistant gloves.

Ensure exhaust ventilation is operating before running production.

Lockout/tagout before entering the oven tunnel for service.

Nitrogen inerting can cause asphyxiation - ensure proper room ventilation.

Common Issues & Troubleshooting

| Symptom | Recommended Action |
| --- | --- |
| Cold solder joints | Verify profile peak temperature and time-above-liquidus. |
| Conveyor stalls | Check chain tension and drive motor; clear jams. |
| Zone not reaching setpoint | Inspect heating element and SSR for that zone. |
| Excess flux residue | Clean oven; check flux management filter saturation. |

Contoso Electronics internal maintenance documentation. For service requests, contact smt.line.support@contoso.com | ext. 4501 or submit a work order through the maintenance portal.