Contoso Electronics

Equipment Reference & Maintenance Guide

Solder Paste Stencil Printer

Contoso PrintPro SP-700

Asset Identification

| Asset ID | CE-SPP-2000 |
| --- | --- |
| Category | SMT Line |
| Manufacturer | Contoso Assembly Tools |
| Location | SMT Line 1 - Position 2 |
| Installation Date | 2021-11-30 |
| Warranty Expiry | 2026-11-30 |
| Support Contact | smt.line.support@contoso.com ¦ ext. 4501 |

Overview

The Contoso PrintPro SP-700 is an automatic solder paste stencil printer that deposits precise volumes of solder paste onto PCB pads before component placement. It features vision alignment, programmable squeegee control, and automated stencil cleaning.

Technical Specifications

| Parameter | Value |
| --- | --- |
| Print Area | Up to 510 x 460 mm |
| Alignment Accuracy | ±12.5 µm @ 6σ |
| Print Force | 1 - 20 kg programmable |
| Squeegee | Metal blade, dual-direction |
| Cycle Time | < 8 seconds (excl. print stroke) |
| Cleaning | Automatic dry/wet/vacuum |
| Power | 230 VAC, 50/60 Hz, 1.5 kW |

Preventive Maintenance Schedule

| Interval | Task |
| --- | --- |
| Each shift | Check paste roll condition and squeegee blades. |
| Daily | Clean stencil underside; verify vision alignment. |
| Weekly | Clean rails and support pins; check vacuum. |
| Monthly | Recalibrate print pressure and camera; inspect blades. |

Safety Guidelines

Follow ESD procedures when loading boards.

Solder paste contains hazardous metals - wear gloves and wash hands.

Keep hands clear of moving squeegee head and print carriage.

Use approved solvents in the automatic cleaner.

Common Issues & Troubleshooting

| Symptom | Recommended Action |
| --- | --- |
| Insufficient paste deposit | Check aperture blockage; adjust print speed/pressure. |
| Smearing/bridging | Clean stencil more frequently; verify gasketing and separation speed. |
| Misalignment | Recalibrate fiducial cameras; verify board support. |
| Inconsistent volume | Inspect squeegee wear; verify paste viscosity and roll size. |

Contoso Electronics internal maintenance documentation. For service requests, contact smt.line.support@contoso.com | ext. 4501 or submit a work order through the maintenance portal.